# Pneumatic Launcher — Complete Analysis

# Input Parameters

Charge chamber volume: 1.0 mL

Working pressure (gauge): 10.0 bar

Barrel ID: 52 mm; Length: 700 mm

# Theoretical Energy Estimates

Ideal isothermal work: 2637.7 J

Isothermal formula: W = P_0 V_0 ln(P_0 / P_f)

# Representative Payload Scenarios

| Mass (kg) | Efficiency (%) | Muzzle velocity (m/s) | Energy (J) |
| --- | --- | --- | --- |
| 0.20 | 25 | 81.2 | 659 |
| 0.20 | 50 | 114.8 | 1319 |
| 0.20 | 75 | 140.7 | 1978 |
| 0.50 | 25 | 51.4 | 659 |
| 0.50 | 50 | 72.6 | 1319 |
| 0.50 | 75 | 89.0 | 1978 |
| 1.00 | 25 | 36.3 | 659 |
| 1.00 | 50 | 51.4 | 1319 |
| 1.00 | 75 | 62.9 | 1978 |

# Structural Checks (Summary)

6061-T6 yield: 276 MPa; design SF used: 3.0

Calculated required hoop thickness at P0: 0.31 mm

| Thickness (mm) | Hoop stress (MPa) | SF vs yield |
| --- | --- | --- |
| 2.0 | 14.3 | 19.30 |
| 3.0 | 9.5 | 28.95 |
| 5.0 | 5.7 | 48.25 |
| 8.0 | 3.6 | 77.20 |

# Transient 1D Simulation Results

Simulated muzzle velocity: 27.60 m/s

Impulse delivered: 27.60 N·s

Discharge time: 37.5 ms

Peak force (sim): 3282 N; Average force: 736 N

| Metric | Value |
| --- | --- |
| Muzzle velocity (m/s) | 27.60 |
| Impulse (N·s) | 27.60 |
| Discharge time (ms) | 37.5 |
| Peak force (N) | 3282 |
| Average force (N) | 736 |

# Parametric Sweep Summary (sample)

| orifice_d_m | Vc_m3 | Cd | muzzle_v | impulse | t_end_s | peak_F |
| --- | --- | --- | --- | --- | --- | --- |
| 0.0060 | 0.0005 | 0.6000 | 7.7771 | 7.7771 | 0.0955 | 2185.8 |
| 0.0060 | 0.0005 | 0.7000 | 9.0378 | 9.0378 | 0.0840 | 2251.3 |
| 0.0060 | 0.0005 | 0.8000 | 10.2887 | 10.2887 | 0.0755 | 2245.8 |
| 0.0060 | 0.0008 | 0.6000 | 7.7903 | 7.7903 | 0.0954 | 2187.4 |
| 0.0060 | 0.0008 | 0.7000 | 9.0630 | 9.0630 | 0.0837 | 2252.8 |
| 0.0060 | 0.0008 | 0.8000 | 10.3334 | 10.3334 | 0.0752 | 2247.5 |
| 0.0060 | 0.0010 | 0.6000 | 7.7940 | 7.7940 | 0.0953 | 2187.9 |
| 0.0060 | 0.0010 | 0.7000 | 9.0699 | 9.0699 | 0.0837 | 2253.3 |
| 0.0060 | 0.0010 | 0.8000 | 10.3454 | 10.3454 | 0.0751 | 2248.0 |
| 0.0060 | 0.0012 | 0.6000 | 7.7963 | 7.7963 | 0.0953 | 2188.3 |
| 0.0060 | 0.0012 | 0.7000 | 9.0741 | 9.0741 | 0.0836 | 2253.6 |
| 0.0060 | 0.0012 | 0.8000 | 10.3526 | 10.3526 | 0.0751 | 2248.4 |
| 0.0080 | 0.0005 | 0.6000 | 13.5262 | 13.5262 | 0.0616 | 2386.8 |
| 0.0080 | 0.0005 | 0.7000 | 15.4529 | 15.4529 | 0.0562 | 2456.4 |
| 0.0080 | 0.0005 | 0.8000 | 17.0951 | 17.0951 | 0.0524 | 2731.4 |
| 0.0080 | 0.0008 | 0.6000 | 13.6793 | 13.6793 | 0.0612 | 2390.3 |
| 0.0080 | 0.0008 | 0.7000 | 15.6795 | 15.6795 | 0.0558 | 2457.1 |
| 0.0080 | 0.0008 | 0.8000 | 17.3920 | 17.3920 | 0.0519 | 2732.4 |
| 0.0080 | 0.0010 | 0.6000 | 13.7203 | 13.7203 | 0.0611 | 2391.4 |
| 0.0080 | 0.0010 | 0.7000 | 15.7440 | 15.7440 | 0.0556 | 2457.3 |
| 0.0080 | 0.0010 | 0.8000 | 17.4802 | 17.4802 | 0.0517 | 2732.8 |
| 0.0080 | 0.0012 | 0.6000 | 13.7445 | 13.7445 | 0.0610 | 2392.2 |
| 0.0080 | 0.0012 | 0.7000 | 15.7848 | 15.7848 | 0.0555 | 2457.4 |
| 0.0080 | 0.0012 | 0.8000 | 17.5377 | 17.5377 | 0.0516 | 2733.0 |
| 0.0100 | 0.0005 | 0.6000 | 19.0088 | 19.0088 | 0.0486 | 2989.5 |
| 0.0100 | 0.0005 | 0.7000 | 20.8232 | 20.8232 | 0.0455 | 2976.9 |
| 0.0100 | 0.0005 | 0.8000 | 22.3461 | 22.3461 | 0.0432 | 2834.0 |
| 0.0100 | 0.0008 | 0.6000 | 19.3993 | 19.3993 | 0.0480 | 2991.4 |
| 0.0100 | 0.0008 | 0.7000 | 21.3281 | 21.3281 | 0.0449 | 2981.3 |
| 0.0100 | 0.0008 | 0.8000 | 22.9690 | 22.9690 | 0.0426 | 2841.3 |

# Plots

vel_vs_orifice_Vc_500uL.png

vel_vs_orifice_Vc_800uL.png

vel_vs_orifice_Vc_1000uL.png

vel_vs_orifice_Vc_1200uL.png

peakF_vs_orifice_Vc_1000uL.png

impulse_vs_orifice_Vc_1000uL.png

# Engineering Drawing

Embedded preview:

# Flange and Bolt Check

## Flange Assumptions

| Parameter | Value |
| --- | --- |
| n_bolts | 6 |
| pcd_m | 0.08 |
| bolt_dia_m | 0.008 |
| bolt_material_shear | 240000000.0 |

## Flange Check Result

| Metric | Value |
| --- | --- |
| n_bolts | 6 |
| cap_per_bolt_N | 12063.715789784805 |
| bolts_needed | 1 |

## Worst-case Sweep Result

| Metric | Value |
| --- | --- |
| orifice_d_m | 0.018 |
| Vc_m3 | 0.0005 |
| Cd | 0.8 |
| muzzle_v | 31.14752690198767 |
| impulse | 31.14752690198767 |
| t_end_s | 0.03339999999999993 |
| peak_F | 7384.324205739658 |

## Bolt Shear Check (endcap)

| Bolt | Capacity (N) | Bolts required |
| --- | --- | --- |
| M6 | 4800 | 1 |
| M8 | 8784 | 1 |
| M10 | 13920 | 1 |

# Formulas and Conclusion

Key formulas used in this analysis:

Isothermal work: W = P_0 V_0 ln(P_0 / P_f)

Hoop stress (thin-wall): \sigma_{hoop} = P\; r / t

Required thickness (hoop): t = P\; r / \sigma_{allowable}

Axial stress (thin-wall): \sigma_{axial} = P\; r / (2\; t)

Impulse and average force: I = m * v; F_avg ≈ I / t_discharge (crude estimate)

Using 6061-T6 (yield ≈ 276 MPa) and safety factor 3.0, allowable stress ≈ 92.0 MPa.

Calculated required hoop thickness at working pressure (10.0 bar gauge) is ≈ 0.31 mm.

Recommendation: select a standard tube/wall thickness equal to or greater than the calculated required thickness, and verify endcap/flange/bolt margins separately. Use ASME/EN pressure-vessel rules or certified fittings for the chamber when safety is critical.

# Barrel–Payload Geometric Compatibility Constraint

The payload–sabot assembly shall fit within the barrel internal diameter under all operating conditions without interference.

## Core Constraint

Clearance condition:

    D_eq + 2·C_sab ≤ D_barrel,min

where:
  D_eq        = √(W² + H²)   (worst-case diagonal envelope of payload cross-section)
  C_sab       = sabot radial clearance per side
  D_barrel,min = D_barrel,nom − manufacturing tolerance

## Numerical Check

Given:
  W = 40 mm,  H = 45 mm  ⇒  D_eq = √(40² + 45²) = 60.2 mm
  C_sab = 1.5 mm  ⇒  2·C_sab = 3.0 mm
  D_barrel,nom = 52 mm,  tolerance = ±0.3 mm
  ⇒  D_barrel,min = 51.7 mm

Check:
  D_eq + 2·C_sab = 60.2 + 3.0 = 63.2 mm
  63.2 mm  >  51.7 mm  ⇒  INTERFERENCE if payload is misaligned in barrel

## Interpretation

The payload does not satisfy the worst-case diagonal envelope constraint. Unconstrained rotation inside the barrel will result in interference (jamming). A sabot or alignment feature is mandatory to constrain the payload orientation and ensure the presented envelope does not exceed D_barrel,min.

## Design Requirements

Requirement G-01:
The sabot shall enforce axial alignment such that payload rotation is constrained and the effective envelope presented to the barrel remains within D_barrel,min.

Requirement G-02:
Total radial clearance (including sabot) shall be limited to 1.0–1.5 mm per side to prevent tilt-induced interference while maintaining acceptable friction and ease of loading.

# Governing ODE System for Launcher Simulation

The launcher’s internal ballistics are modeled as a coupled system of ordinary differential equations (ODEs), solved numerically at each timestep. The state vector is:

    s(t) = [x, v, P, m]

where:
  x = projectile position
  v = projectile velocity
  P = chamber pressure
  m = mass of gas in chamber

The system:
1. Projectile motion (Newton’s law):
   dv/dt = (P * A_bore - F_friction) / m_payload
2. Kinematics:
   dx/dt = v
3. Gas mass balance:
   dm/dt = -m_dot_out
4. Pressure evolution (ideal gas, real-gas correction applied in code):
   P = m * R * T / V(t)
   dP/dt = (R * T / V) * dm/dt - (m * R * T / V^2) * dV/dt
5. Chamber volume change (moving projectile):
   V(t) = V0 + A_bore * x(t)
   dV/dt = A_bore * v
6. Valve mass flow (critical):
   Choked: m_dot_out = Cd * A_orifice * P * sqrt(gamma / (R * T)) * (2/(gamma+1))^((gamma+1)/(2*(gamma-1)))
   Subsonic: m_dot_out = f(Cv, P_up, P_down)

Key insight: There is no single closed-form velocity equation. The exit velocity is obtained by integrating this ODE system over time.

This system is implemented in simulate_transient() in scripts/gen_doc.py.